# Lane to Byte Mapping Functional Design Specifications

UCIe 3.0 Physical Layer

Version 1

Block Owner

ENG. Karim Waseem

Authors

Mahmoud Hesham

This Page is left Blank Intentionally

# Table of Contents

1	Table of Contents	1

2	Revision History	4

3	Overview	5

4	Operation and Description	5

4.1	Digital Interface	5

4.1.1	Parameters Names	5

4.1.2	Ports Names	5

4.2	Functional Description	7

4.2.1 Input Interface and Control Signals	7

4.2.2 Lane Mapping Decoder	7

4.2.3 Enable Gating Logic	7

4.2.4 Data Multiplexing and Grouping	7

4.2.5 Reordering block	8

4.2.6 Output Selection and Formatting	8

4.3	Block Diagram	9

4.4	Timing Diagram	9

4.5	Verification Requirements	10

# Revision History

| Version | Date | Author(s) | Revision Notes | Owner Approval |
| --- | --- | --- | --- | --- |

# Overview

This block implements a configurable multi-lane data aggregation and packing unit.
It receives 16 parallel input lanes, each 64 bits wide, and assembles them into a single 2048-bit output word.

The block supports multiple lane mapping modes (x16, x8, x4) selected via a lane map control code. Based on the selected mode, input lanes are conditionally enabled, multiplexed, and accumulated using mode-specific shift registers. A final output multiplexer selects the correctly packed data path and drives the output interface.

# Operation and Description

## Digital Interface

### Parameters Names

| Parameter Name | Default | Description |
| --- | --- | --- |
| pDATA_IN_WIDTH | 64 | Input data word width |
| pDATA_OUT_WIDTH | 2048 | Output data word width |

### Ports Names

| Port Name | Port Width | Port Type | Description |
| --- | --- | --- | --- |
| i_lane_0 | pDATA_IN_WIDTH | input | Parallel input data lanes that will be packed. Each lane provides a 64-bit data word at each cycle |
| i_lane_1 |  | input |  |
| i_lane_2 |  | input |  |
| i_lane_3 |  | input |  |
| i_lane_4 |  | input |  |
| i_lane_5 |  | input |  |
| i_lane_6 |  | input |  |
| i_lane_7 |  | input |  |
| i_lane_8 |  | input |  |
| i_lane_9 |  | input |  |
| i_lane_10 |  | input |  |
| i_lane_11 |  | input |  |
| i_lane_12 |  | input |  |
| i_lane_13 |  | input |  |
| i_lane_14 |  | Input |  |
| i_lane_15 |  | input |  |
| i_lane_map_code [2:0] | 3 | input | Lane Map Code. Selects lane mapping configuration as listed in 4.1.3. |
| i_clk | 1 | input | Clock used in design |
| i_enable | 1 | input | Enable for lane to byte mapping block |
| i_reset | 1 | input | Reset for the system |
| i_lane_map_code | 3 | input | Control signals determine functional lanes |
| o_pl_valid | 1 | input | Flag to controllers that indicates data packed and valid |
| o_pl_data | pDATA_OUT_WIDTH | output | Output packed data |

    - Standard Package Logical Lane Map

| Lane Map Code | Functional Lanes | Decodings{mux_8,mux_4,x16_en,x8_en,x4_en} |
| --- | --- | --- |
| 000b | None (Degrade not possible) | 0 0 1 0 0 |
| 001b | Logical Lanes 0 to 7 | 0 0 0 1 0 |
| 010b | Logical Lanes 8 to 15 | 1 0 0 1 0 |
| 011b | 0 - 15 | 0 0 1 0 0 |
| 100b | Logical Lanes 0 to 3 | 0 0 0 0 1 |
| 101b | Logical Lanes 4 to 7 | 0 1 0 0 1 |

mux_8 : determines which lanes will be taken (0 to 7) or (8 to 15)

mux_4: determines which lanes will be taken (0 to 3) or (4 to7)

en_x16 : enable signal for x16 lanes shift registers

en_x8 : enable signal for x8 lanes shift registers

en_x4 : enable signal for x4 lanes shift registers

## Functional Description

## 4.2.1 Input Interface and Control Signals

## Sixteen parallel input data lanes (i_lane_0 to i_lane_15) provide 64-bit data words.

## All inputs are sampled on the rising edge of i_clk when i_enable is asserted.

## The i_reset signal clears all internal storage elements and control logic.

## 4.2.2 Lane Mapping Decoder

## The i_lane_map_code input is decoded to generate mode-specific enable signals:

## x16_en , x8_en , x4_en

## mux_x4 , mux_x8

## These enable signals control:

## Which shift registers are active

## Which multiplexers are enabled

## The final output selection

## Only one lane mapping mode is active at any given time.

## 4.2.3 Enable Gating Logic

## Each mapping mode enable is logically ANDed with i_enable.

## This ensures that no data is captured or shifted unless the block is explicitly enabled.

## 4.2.4 Data Multiplexing and Grouping

## Depending on the selected mode, input lanes are grouped using multiplexers:

## x16 Mode

## Each input lane feeds its own 128-bit shift register.

## All 16 lanes operate independently and in parallel.

## x8 Mode

## Input lanes are selected using 2-to-1 multiplexers.

## Each pair feeds a 256-bit shift register.

## Eight  shift registers operate in parallel.

## x4 Mode

## Input lanes are selected using 2-to-1 multiplexers.

## Each group feeds a 512-bit shift register.

## Four such shift registers operate in parallel.

## The multiplexers ensure correct lane selection and alignment for each mode.

## 4.2.5 Shift Register Accumulation

## Shift registers accumulate incoming 64-bit data words on each clock cycle.

## Data is shifted until the full register width is filled.

## The number of clock cycles required to complete accumulation depends on the active lane mapping mode.

## All shift registers are synchronously reset by i_reset.

## 4.2.5 Reordering block

- Concatenates output data from shift register in its order

- It works in 3 modes x16 , x8 and x4

## 4.2.6 Output Selection and Formatting

## Outputs of the active shift register bank are routed to a final wide multiplexer.

## The multiplexer selects the correct 2048-bit packed data based on the active lane mapping mode.

## The selected data is driven on the o_data output bus.

## Block Diagram

## Timing Diagram

x16 lane mode:

x8 Lane mode:

Lanes (0 to 7):

Lanes(7 to 15):

x4 Lane mode:

Lanes (0 to 3):

Lanes(4 to 7):

## Verification Requirements

4.5 Verification Requirements

The following verification scenarios shall be implemented to ensure correct functionality:

4.5.1 Reset Verification

- Assert i_reset and verify that:

  - All shift registers are cleared

  - o_data is driven to zero or an invalid state

- Deassert reset and confirm normal operation resumes.

4.5.2 Enable Control Verification

- Verify that data shifting occurs only when i_enable is asserted.

- Confirm that deasserting i_enable freezes internal state.

4.5.3 Lane Mapping Mode Verification

- Independently verify each lane mapping mode:

  - x16 mode

  - x8 mode

  - x4 mode

- Confirm correct enable activation and correct mux selection per mode.

4.5.4 Data Packing and Ordering

- Apply known data patterns on input lanes.

- Verify correct lane ordering, alignment, and bit placement within o_data.

- Confirm valid data flag asserted when data is ready.